Project Report #3

Computer Architecture

김경민

2022171063

- Explain the structure of the provided skeleton and how it works as per your understanding

The goal of this code scheme is to build a cache able to simulate and track how does it handle cache misses and hits, using a write-back policy. We’re utilizing the l1cache structure declared on cache.h. It has set bits, data bits, and ways that is crucial to designing cache. Each lines of cache structure has a data bits of age that enables us to decide where to place a new data block inbound. Cache.c uses structures predetermined at cache.h. cache_simulator is the driver, putting all codes together to yield a sensible output.

- Describe how you implemented the build_cache() and access_cache() codes

build_cache() method initializes the cache structure. It sets how many data bits and set bits the cache structure is going to use, and how many ways it would utilize. It allocates appropriate register spaces and sets all tags to zero.

Access_cache() method runs on LRU replacements and write-back policy. It parses address(tag, index and block offset from the 32 bit block) and search through the target tag. If it matches with the data block in the cache, it increments hit counter by one and reset hit line to 0(age counter/0 is most recently used). Setting dirty bit helps register to write back the updated value to the memory. If it misses, it increments miss counter, and find the replacement candidate.

- What you learned from this assignment

Little did I know modern computers depend on predictions and speculations to enable fast reading and writing. I am still astonished by how people come up with new ways for faster processing, even though some trade-offs and inaccuracies take place. I also love that instead of building a whole cache in expensive and fast hardware, they compromised L2/L3 cache to cut budgets. I personally feel the cache miss handling would get better if we apply some big data analysis and AI in this matter.